|  | TRS Ceramics, Inc. Suite J, 2820 East College Avenue, State College, PA 16801 phone: 814/238-7485 fax: 814/238-7539 info@trsceramics.com |
| --- | --- |

MATERIAL SAFETY DATA SHEET

Section I - Product Identification

Manufacturer’s Name:	TRS Ceramics, Inc.

2820 East College Avenue, Suite J

State College, PA 16801

Telephone:  (814) 238-7485

Fax:  (814) 238-7539

Chemical Names:	Lead-Zirconate-Titanate (PZT)

Trade Names:	TRS 300FG

Section II - Ingredients / Exposure Information

| Material or Component | CAS Number | Reference Weight % | TLV-TWA mg/X3 | TLV-STEL mg/X3-8 hr | PEL mg/X3-8 hr |
| --- | --- | --- | --- | --- | --- |
| Lead Monoxide (PbO)* | 1317-36-8 | 60-65 | 0.15 as Pb | 0.45 as Pb | 0.05 as Pb |
| Zirconium Dioxide (ZrO2) | 1314-23-4 | 20-25 | 5.00 as Zr | 10.00 as Zr | 5.00 as Zr |
| Titanium Dioxide (TiO2) | 13463-67-7 | 10-15 | 10.00 | 10.00 | 5.00 respirable 15.00 total |
| Iron Oxide (Fe2O3) | 1309-37-1 | <1 | 5.00 | -- | 10.00, fume |
| Strontium Carbonate (SrCO3) | 1633-05-2 | 1-5 | NE | -- | NE |

NE = Not Established

* SARA Section 313 Supplier Notification - These chemicals are subject to the reporting requirements of Section 313 of the Emergency Planning and Community Right-to-Know Act of 1986 and of 40 CFR 372.

Material is a lead compound.  Refer to Federal Register Vol. 43 #220, pgs. 53007-53014, November 14, 1978.  Standard (OSHA) - Occupational Exposure to lead.  Also, Code of Federal Regulation CFR Title 29 1910.1025.

Section III - Physical Data

| Boiling Point @ 760mm Hg: | N/A |  | Specific gravity (H2O=1): | 7.6 - 8.2 |
| --- | --- | --- | --- | --- |
| Melting Point: | 1310 to 1350°C |  | Solubility in H2O (g/L): | 0.05 |
| Vapor Pressure (mmHg): | 10 @ 1985°F |  | Evaporation Rate: | N/A |
| % Volatiles by Volume: | N/A |  | Molecular Weight: | 318.1 to 337.3 |
| Appearance: | Yellow/Tan/Orange |  | Odor: | Odorless |

Section IV - Fire and Explosion Hazard Data

Flammability:  NFPA Flammability Rating = 0   Flash Point:  NA   Autoignition Temperature:  NA   Flammable Limits:  NA   Extinguishing Media:  Water Fog or Flood, CO2 and Dry Chemical.

Special Fire Fighting Procedures:  Wear full body protective clothing and full-face piece, self-contained breathing apparatus operated in positive-pressure mode.

Unusual Fire and Explosion Hazard:  Fume, vapor and/or dust may occur and are considered toxic and respiratory irritants.  The product or dust can react vigorously with strong oxidizing agents.  Refer to Fire Protection Guide on Hazardous Materials by N.F.P.A. for specific individual problem combinations.

Section V - Reactivity Data

Reactivity:	NFPA Exis Rating = 1

Stability:	Stable to 500°C

Conditions to Avoid:	Do not heat in the presence of strong oxidizers.

Hazardous Decomposition Products:	High temperatures or fire may produce lead oxide fume, vapor, or dust.

Hazardous Polymerization:	N/A

Section VI - Spill or Leak Procedures

Steps to be taken in case material is spilled or broken:  Wear gloves, goggles, and respirator.  Do not breathe or ingest dust.  Exclude all individuals not wearing protective equipment from the spill area until clean up is completed.  Clean the area of spill, returning all material possible to container.  For a small spill, use a vacuum with a high efficiency filter (HEPA).  Do Not Broom Sweep.  Do Not Use Compressed Air.  Recovered material should be stored in dry containers for later disposition.

Waste Disposal Method:  Disposal of waste and hazardous material should be handled in a manner, which complies with local, state, and federal Environmental Protection Agency regulations.  It is preferable to send waste material to an approved recycling facility.  City, state, and federal regulations must be followed at all times.

Neutralizing Chemicals:  N/A

Section VII - Health Hazard Information

Routes of exposure when handling or processing  (Health Hazard:  NFPA EXIS Rating = 2)

Mode of entry into body:

1.	By Inhalation of dust or fumes, the respiratory system may be irritated and both acute and chronic effects may result, as described below.

- By Ingestion of dust trapped in the upper respiratory tract or introduced into the mouth on food, tobacco, fingers or other objects, both acute and chronic effects can result.

3.	By Skin Contact of dust may cause irritation.

4.	Eye Contact of dusts or fumes may cause irritation.

Effects from Acute Overexposure:  Lead intoxication will occur with accompanying symptoms of constipation, sleep disturbance, fatigue, headache, loss of appetite.  Where inhalation is severe from heavy dusting or a large quantity is ingested and left untreated, colic, anemia, vomiting and neuritis will follow as evidenced by intense periodic cramps, aching bones and muscles, uncoordinated body movements.  Worst-case situations could result in convulsions, stupor, coma, and encephalopathy.

Titanium Dioxide may have a temporary irritating effect to the mucus membranes if excessive exposure occurs.  Zirconium Dioxide is an irritant to the respiratory tract.  Iron Oxide, Strontium Carbonate, and Niobium Pentoxide is an irritant to the eyes, skin, and respiratory tract.

Effects from Chronic Overexposure:  Early effects of chronic overexposure to lead are difficult to detect, but symptoms include persistent fatigue, sleep disturbance, headache, aching bones and muscles, constipation, abdominal pains and loss of appetite.  Prolonged ingestion may be associated with intense periodic cramps and constipation, nausea and vomiting.  Excessive exposure may affect blood, nervous and digestive systems.  Synthesis of hemoglobin is inhibited and can result in anemia.  If left untreated, neuromuscular dysfunction, possible paralysis, and encephalopathy can result.  Unusual occurrence of symptoms should prompt immediate contact of a physician. For industrial exposure, a worker’s lead accumulation can be detected by an increase in blood lead above the base level established upon the employee’s entry to the workplace.

Excessive exposure to Zirconium Oxide may cause damage to teeth and ulceration of mucus membranes.

Iron compounds may cause vomiting, diarrhea, pink urine, black stool, and liver damage.  May cause damage to kidneys.  Irritating to the respiratory tract, they may cause pulmonary fibrosis if dusts are inhaled.

Emergency and First Aid Procedures:

Inhalation:	Remove from exposure.  Get medication attention if experiencing effects of acute overexposure.

Ingestion:	Get immediate medical attention.  Call a physician.

Skin:	Wash thoroughly with soap and water.

Eyes:	Flush with copious quantities of water.  Get immediate medical attention.

NOTES TO PHYSICIAN:  	Lead and its inorganic compounds are neurotorius, which may produce peripheral neuropathy.  For an overview of the effects of lead exposure, consult Occupational Safety and Health Administration A11.A of Occupational Exposure to Lead (29CFR1910.1025).  “A Guide for Physicians, Health Maintenance of Workers Exposed to Inorganic Lead” is available from Lead Industries Association, Inc., 292 Madison Ave., New York, NY 10017.

Section VIII - Special Protection

Specific Personal Protective Equipment:

Respirator:	As specified by 29CFR1910.1025(f) of the Federal Occupational Safety and Health Administration Standard for Occupational Exposure to Lead -- When handling, use a dust/fume respirator with NIOSH/WSHA approval.  Respirator must be worn if TLV is exceeded.

Protective Gloves:	Protective gloves recommended.

Eye Protection:   Protective glasses recommended.

Other Clothing & Equipment:	Hardhat, safety boots, and other safety equipment should be worn as appropriate for the industrial environment.  Personal clothing, including shoes should be protected from contamination.

Ventilation Requirements:

Ventilation as described in the Industrial Ventilation Manual, by ACGIH shall be provided where exposures exceed PEL or TLV specified in and in accord with OSHA Standard 29CFR1910.94.  Other local and state regulations may apply.

Section IX - Definitions

| TLV-TWA | Threshold Limit Value-Time Weighted Average (American Conf. of Governmental Industrial Hygienists) |
| --- | --- |
| TLV-STEL | Threshold Limit Value - Short Term Exposure Limit (ACGIH) |
| PEL | Permissible Exposure Limit (OSHA) |

Section X - Special Precautions

Precautions:	There are two major methods of lead absorption; namely inhalation and ingestion.  Most inhalation problems can be prevented with attention to ventilation and respirator use.  Ingestion can be prevented with good hygiene practices.

- Do not inhale or swallow dust.

- Wash thoroughly after handling.

- Use gloves and avoid spilling on hands, face, or body, to prevent secondary contamination by ingesting or inhalation.

- Do not smoke, eat, or apply cosmetics in work area or prior to washing hands.

- Wash thoroughly before entering into eating areas.

- Do not wear any part of work clothing home.  This includes shoes.

- Keep lead materials away from food and food products.

- Keep away from children.

- Do not reuse containers.

- This product is intended for industrial use only.

Section XI - Other Handling and Storage Requirements

Store in dry area where accident contact with acids or bases is not possible.  When a spill releases material into the environment, it may be necessary to file a report of the discharge.  Contact your local environmental authorities to determine what rules apply.  Adhere to all personal protection procedures and ventilation requirements when exposure limits exceed PEL or TLV.

Before using this product, be familiar with information contained in:  The Federal Standard for Occupational Exposure to Lead (29CFR1910.1025), first published in the Federal Register on Tuesday, November 14, 1978, by the Occupational Safety and Health administration, and as also later modified.

Prepared By:

TRS Ceramics, Inc.

2820 East College Avenue, Suite J

State College, PA 16801

Vendee and third persons assume the risk of injury proximately caused by the material if reasonable safety procedures are not followed as provided for in the data sheet, and vendor shall not be liable for injury to vendee or third persons proximately caused by abnormal use of the material even if reasonable safety procedures are followed.

All persons using this product, all persons working in an area where this product is used, and all persons handling this product should be familiar with contents of this data sheet.  This information should be effectively communicated to employees and others who might come in contact with the product.

While the information accumulated and set forth herein is believed to be accurate as of the date hereof, TRS Ceramics, Inc. makes no warrant with respect thereto and disclaims all liability from reliance thereon.  Recipients are advised to confirm in advance of need that the information is current, applicable, and suitable for their particular circumstances.